Production standard: ETFPC - with gradual DC brake

| Production standard: ETFPC - with gradual DC brake |  |
| --- | --- |
| Mounting - arrangement | B3 |
| Terminal box position | PST DX E - Left side near fan cover |
| Paint finish | •; non verniciato. |
| Terminal box type | 2CA - 2 alluminium components (box + cover) |
| Connections |  |
| Rated voltage | 230/400V - 50Hz ( Pn<= 3 kW) 400/690V - 50Hz ( Pn> 3 kW) |
| Brake power supply (supplied disconnected) | 230V ( Pn<= 3 kW) 400V ( Pn> 3 kW) |
| Protection rating | IP54 - Protection against dust, water splashes from all directions |
| Duty | S6- 60% - periodic constant service noload 6min/4min on load |
| Winding insulation | F |
| Bearings | 2RS <> |
| End-shield assembly | Tie rods |
| Fan cover | E48 aluminium - E63/E80 plastic |

| ETFPC Low centre motor with FPC brake |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 3000 min-1 2 poles 50 Hz - V=230/400V @ Pn<= 3 kW , V=400/690V @ Pn> 3 kW |  |  |  |  |  |  |  |  |  |  |  |  |
| Size | Pn | Pn | n | Inom | cosφ | η | Mn | Mm/Mn | Ms/Mn | Is/In | J | Weight |
|  | kW | Hp | min-1 | A |  | % | Nm |  |  |  | Kgm2 | Kg |
| E48Sb | 0,55 | 0,75 | 2700 | 1,4 | 0,84 | 67 | 1,94 | 2 | 2,1 | 3 | 0,00057 | 6,6 |
| E48Sc | 0,75 | 1 | 2700 | 1,9 | 0,86 | 66 | 2,65 | 2 | 1,9 | 3,2 | 0,00063 | 7,5 |
| E48Ma | 1,1 | 1,5 | 2700 | 2,6 | 0,83 | 75 | 3,89 | 2,5 | 2,6 | 3,9 | 0,00068 | 9 |
| E48Mb | 1,3 | 1,8 | 2700 | 3,1 | 0,82 | 74 | 4,6 | 2,8 | 2,5 | 4,5 | 0,00075 | 10,5 |
| E48L | 1,5 | 2 | 2700 | 3,6 | 0,82 | 74 | 5,3 | 3,2 | 3,3 | 4,6 | 0,00081 | 11 |
| E63Sa | 1,1 | 1,5 | 2700 | 2,6 | 0,83 | 75 | 3,89 | 2,3 | 2,3 | 4,6 | 0,0017 | 12,9 |
| E63Sb | 1,5 | 2 | 2700 | 3,4 | 0,85 | 75 | 5,3 | 2,6 | 2,5 | 4,3 | 0,00185 | 13,7 |
| E63Sc | 1,85 | 2,5 | 2750 | 4,2 | 0,85 | 75 | 6,42 | 3 | 3 | 4,5 | 0,002 | 14,7 |
| E63La | 2,2 | 3 | 2750 | 4,7 | 0,85 | 80 | 7,64 | 3,2 | 3,2 | 5,5 | 0,00213 | 17,7 |
| E63Lb | 3 | 4 | 2800 | 6,4 | 0,84 | 81 | 10,2 | 3,2 | 3,5 | 6 | 0,00225 | 19,2 |
| E80S | 3 | 4 | 2850 | 6,4 | 0,84 | 81 | 10 | 3,2 | 3,2 | 6 | 0,00283 | 28 |
| E80Sb | 4 | 5,5 | 2890 | 8,8 | 0,8 | 82 | 13,2 | 3,3 | 3,4 | 6,8 | 0,00323 | 30 |
| E80M | 5,5 | 7,5 | 2880 | 11,7 | 0,8 | 85 | 18,2 | 3,6 | 3,5 | 6,8 | 0,00348 | 33,8 |
| E80L | 7,5 | 10 | 2870 | 15 | 0,85 | 85 | 25 | 3,1 | 3,3 | 6 | 0,00398 | 41,3 |

Data Tables

| ETFPC Low centre motor with FPC brake |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1500 min-1 4 poles 50 Hz - V=230/400V @ Pn<= 3 kW , V=400/690V @ Pn> 3 kW |  |  |  |  |  |  |  |  |  |  |  |  |
| Size | Pn | Pn | n | Inom | cosφ | η | Mn | Mm/Mn | Ms/Mn | Is/In | J | Weight |
|  | kW | Hp | min-1 | A |  | % | Nm |  |  |  | Kgm2 | Kg |
| E48S | 0,37 | 0,5 | 1400 | 1,55 | 0,54 | 65 | 2,52 | 3,9 | 4 | 3,5 | 0,00073 | 7,7 |
| E48M | 0,55 | 0,75 | 1300 | 1,7 | 0,7 | 67 | 4,04 | 2,5 | 2,6 | 3,2 | 0,00083 | 9,5 |
| E48L | 0,75 | 1 | 1300 | 2,3 | 0,78 | 60 | 5,51 | 2,2 | 2,3 | 2,2 | 0,00088 | 10,3 |
| E63Sa | 0,75 | 1 | 1380 | 2 | 0,72 | 74 | 5,19 | 2,1 | 2 | 3,5 | 0,0022 | 13,4 |
| E63Sb | 1,1 | 1,5 | 1400 | 2,7 | 0,77 | 76 | 7,5 | 2,2 | 2,3 | 3,9 | 0,00235 | 14,8 |
| E63La | 1,5 | 2 | 1300 | 4 | 0,75 | 73 | 11 | 1,8 | 2 | 3 | 0,0026 | 16,8 |
| E80S | 2,2 | 3 | 1400 | 5 | 0,8 | 80 | 15 | 3 | 2,7 | 5 | 0,00403 | 26 |
| E80M | 3 | 4 | 1370 | 6,8 | 0,8 | 80 | 20,9 | 2,2 | 2,3 | 4,4 | 0,00448 | 32,5 |

Dimension Diagrams

DISTANCE BETWEEN SHAFTS 63/71 mm (E63) THREE PHASE WITH BLADE DISCS (B3)

| Size | A | AB | AC | AA | B | E | F | G | H | HB | L | LB | K | Y | P | AD | AF | AL | LA |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E63S | 120 | 140 | 128 | 80 | 180 | 49 | 10 | 32 | 63 | 71 | 330 | 247 | 10 | 8 | 114 | 125 | 111 | 77 | 20 |
| E63L | 120 | 140 | 128 | 80 | 240 | 49 | 10 | 32 | 63 | 71 | 390 | 307 | 10 | 8 | 114 | 125 | 111 | 137 | 20 |

DISTANCE BETWEEN SHAFTS 80 mm (E80) THREE PHASE WITH BLADE DISCS (B3)

| Size | A | AB | AC | AA | B | E | F | G | H | HB | L | LB | K | Y | P | AD | AF | AL | LA |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E80S | 125 | 160 | 156 | 110 | 230 | 58 | 12 | 46 | 80 | 80 | 421 | 320 | 10 | 10 | 140 | 135 | 111 | 125 | 23 |
| E80M | 125 | 160 | 156 | 110 | 280 | 58 | 12 | 46 | 80 | 80 | 471 | 370 | 10 | 10 | 140 | 135 | 111 | 175 | 23 |
| E80L | 125 | 160 | 156 | 110 | 350 | 58 | 12 | 46 | 80 | 80 | 541 | 440 | 10 | 10 | 140 | 135 | 111 | 245 | 23 |

DISTANCE BETWEEN SHAFTS 63/71 mm (E63) THREE PHASE WITH COUPLING FLANGE (B14)

| Size | A | AB | AC | AA | B | H | HB | L | LB | Y | K | P | N | M | T | S | D | E | F | FB | GA | GD | AD | AF | AL | DB |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E63S | 120 | 140 | 128 | 80 | 180 | 63 | 71 | 321 | 281 | 8 | 10 | 120 | 80 | 100 | 3 | M6 | 19 | 40 | 6 | 30 | 21,5 | 6 | 125 | 111 | 111 | M6 |
| E63L | 120 | 140 | 128 | 80 | 240 | 63 | 71 | 391 | 341 | 8 | 10 | 120 | 80 | 100 | 3 | M6 | 24 | 50 | 8 | 40 | 27 | 7 | 125 | 111 | 171 | M8 |

DISTANCE BETWEEN SHAFTS 80 mm (E80) THREE PHASE WITH COUPLING FLANGE (B14)

| Size | A | AB | AC | AA | B | H | HB | L | LB | K | Y | P | N | M | T | S | D | E | F | FB | GA | GD | AD | AF | AL | DB |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E80S | 125 | 160 | 156 | 110 | 230 | 80 | 80 | 410 | 360 | 10 | 10 | 140 | 95 | 115 | 3 | M8 | 24 | 50 | 8 | 40 | 27 | 7 | 134 | 111 | 165 | M8 |
| E80M | 125 | 160 | 156 | 110 | 280 | 80 | 80 | 470 | 410 | 10 | 10 | 140 | 95 | 115 | 3 | M8 | 28 | 60 | 8 | 50 | 31 | 7 | 134 | 111 | 215 | M10 |
| E80L | 125 | 160 | 156 | 110 | 350 | 80 | 80 | 540 | 480 | 10 | 10 | 140 | 95 | 115 | 3 | M8 | 28 | 60 | 8 | 50 | 31 | 7 | 134 | 111 | 285 | M10 |

# E63 Exploded View

|  | 1; Blade nut. 2; Blade clamping disc. 3; Blade holding disc. 4; Dust seal (V-ring). 5; Washers. 6; End-shield (drive end). 7; Tie rods. 8; Bearing (drive end). 9; Circlip. 10; Shaft. 11; Rotor. 12; Stator assembly. 13; Casing. 14; Bearing (non-drive end). 15; Pre-load washer. 16; End-shield (non-drive end). 17; Cooling fan (PVC). |  | 18; Fan tightening ring. 19; Tie rods nuts. 20; Fan cover. 21; Cable gland. 22; Terminal box – cover. 23; Terminal box fixing screws. 24; IP55 rubber gasket. 25; Terminal box. 26; Terminal block. 27; IP55 rubber gasket. 28; Terminal box base (PVC). 29; Nameplate fixing screws. 30; Nameplate. 31; Shaft key. 32; B14 flange IEC80. 33; B14 flange IEC71. 34; Oil seal. |  | 35; MM type: Capacitor-holder – box. 36; MM type: Capacitor. 37; MM type: capacitor clamp. 38; MM type:IP55 rubber gasket. 39; MM type: capacitor holder – cover. 40; MM type: capacitor holder fixing screws. 41; FPC version: shaft. 42; FPC version: shaft key. 43; FPC version: end-shield (non-drive end). 44; FPC version: brake unit. 45; FPC version: adjustment axial spring. 46; FPC version: cooling fan (cast iron). 47; FPC version: brake adjustment screws. |
| --- | --- | --- | --- | --- | --- |

# E80 Exploded View

|  | 1; Blade nut. 2; Blade clamping disc. 3; Blade holding disc. 4; Dust seal (V-ring). 5; Tie rods nuts. 6; Washers. 7; Bearing fixing disc tightening screws. 8; Cast iron end-shield (drive end). 9; Bearing (drive-end). 10; Circlip. 11; Tie rods. 12; Shaft. 13; Rotor. 14; Stator assembly. 15; Casing. 16; Bearing (non-drive end). 17; Pre-load washer. |  | 18; Cast iron end-shield (non-drive end). 19; Cooling fan (PVC). 20; Fan tightening ring. 21; Fan cover. 22; Cable gland. 23; Terminal box – cover. 24; Terminal box fixing screws. 25; IP65 rubber gasket. 26; Terminal box. 27; Terminal block. 28; IP55 rubber gasket. 29; terminal box plate (PVC). 30; Nameplate fixing screws. 31; Nameplate. 32; Shaft key. 33; B14 flange IEC90. 34; Oil seal. |  | 35; MM type: IP55 rubber gasket. 36; MM type: Capacitor-holder – box. 37; MM type: Capacitor. 38; MM type: Capacitor clamp. 39; MM type: Capacitor-holder – cover. 40; MM type: capacitor holder fixing screws. 41; FPC version: shaft. 42; FPC version: shaft key. 43; FPC version: end-shield (non-drive end). 44; FPC version: brake unit. 45; FPC version: adjustment axial spring. 46; FPC version: cooling fan (cast iron). 47; FPC version: brake adjustment screws. |
| --- | --- | --- | --- | --- | --- |

# Brake Data Tables

| Size | Braking torque | Power | Closing Time | Release time | Braking gap |
| --- | --- | --- | --- | --- | --- |
|  | Nm | W | ms | ms | mm |
| E63 | 4 | 18 | 40 | 100 | 0,2 |
| E80 | 7 | 25 | 60 | 150 | 0,2 |