# Production standard: MMDSG - cap start / cap run

| Production standard: MMDSG - cap start / cap run |  |
| --- | --- |
| Mounting - arrangement | B3 |
| Terminal box position | ANT BS - On top (BS) |
| Paint finish | •; 5010 BLUE. |
| Terminal box type | PCN - Capacitor Holder - Box |
| Connections |  |
| Rated voltage | 230V - 50Hz |
| Protection rating | IP54 - Protection against dust, water splashes from all directions |
| Duty | S1 - continuous duty at full load |
| Winding insulation | F |
| Bearings | 2 RS <> |
| End-shield assembly | Bosses and bolts |
| Fan cover | Galvanized steel sheet |

Data Tables

| MMDSG Single-phase cap start/cap run motors 3000 min-1 2 poles 50 Hz V=230V |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Size | Pn | Pn | n | Inom | cosφ | η | Mn | Mm/Mn | Ms/Mn | Is/In | Ca | Cm | J | Weight |  |
|  | kW | Hp | min-1 | A |  | % | Nm |  |  |  | μF | μ | Kgm2 | Kg |  |
| M71a | 0,37 | 0,5 | 2730 | 3,4 | 0,89 | 53 | 1,3 | 0 | 1,3 | 2,8 | 63-80 | 12 | 0,00032 | 7 |  |
| M71b | 0,55 | 0,75 | 2730 | 4,3 | 0,92 | 61 | 1,92 | 0 | 1,4 | 2,8 | 63-80 | 16 | 0,00038 | 8,3 |  |
| M80a | 0,75 | 1 | 2770 | 5,1 | 0,94 | 68 | 2,58 | 0 | 1,5 | 3,6 | 63-80 | 20 | 0,00066 | 10 |  |
| M80b | 1,1 | 1,5 | 2750 | 7,1 | 0,96 | 70 | 3,82 | 0 | 1,6 | 3,5 | 80-100 | 25 | 0,00083 | 11,8 |  |
| M80c | 1,5 | 2 | 2700 | 9,8 | 0,95 | 70 | 5,3 | 0 | 1,6 | 3,3 | 80-100 | 35 | 0,00092 | 12,5 |  |
| M90S | 1,5 | 2 | 2750 | 9 | 0,98 | 75 | 5,2 | 0 | 1,9 | 3,7 | 100-125 | 40 | 0,00114 | 15 |  |
| M90La | 1,85 | 2,5 | 2750 | 10,7 | 0,99 | 74 | 6,4 | 0 | 2 | 3,7 | 100-125 | 50 | 0,0013 | 16,8 |  |
| M90Lb | 2,2 | 3 | 2740 | 13,4 | 0,99 | 72 | 6,7 | 0 | 1,9 | 3,2 | 125-156 | 60 | 0,0013 | 16,8 |  |
| M100L | 2,5 | 3,5 | 2850 | 14,7 | 0,99 | 73 | 8,4 | 0 | 1,8 | 4 | 125-156 | 80 | 0,00246 | 22,4 |  |

| MMDSG Single-phase cap start/cap run motors 1500 min-1 4 poles 50 Hz V=230V |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Size | Pn | Pn | n | Inom | cosφ | η | Mn | Mm/Mn | Ms/Mn | Is/In | Ca | Cm | J | Weight |  |
|  | kW | Hp | min-1 | A |  | % | Nm |  |  |  | μF | μ | Kgm2 | Kg |  |
| M71 | 0,37 | 0,5 | 1300 | 3,2 | 0,92 | 55 | 2,72 | 0 | 1,5 | 2,3 | 63-80 | 16 | 0,00086 | 8 |  |
| M80a | 0,55 | 0,75 | 1360 | 3,7 | 0,98 | 64 | 3,86 | 0 | 1,3 | 2,7 | 63-80 | 16 | 0,00142 | 9 |  |
| M80b | 0,75 | 1 | 1400 | 5,3 | 0,94 | 65 | 5,1 | 0 | 1,3 | 3,5 | 63-80 | 20 | 0,00184 | 10,4 |  |
| M90S | 1,1 | 1,5 | 1380 | 6,7 | 0,98 | 71 | 7,6 | 0 | 1,3 | 3,3 | 80-100 | 35 | 0,00185 | 13,4 |  |
| M90La | 1,5 | 2 | 1360 | 9,3 | 0,96 | 73 | 10,8 | 0 | 1,5 | 3 | 100-125 | 45 | 0,00247 | 16,2 |  |
| M90Lb | 1,85 | 2,5 | 1360 | 12,1 | 0,91 | 73 | 13 | 0 | 1,5 | 3,1 | 100-125 | 55 | 0,00268 | 18,2 |  |
| M100La | 1,85 | 2,5 | 1360 | 12,2 | 0,94 | 70 | 13 | 0 | 1,6 | 3 | 100-125 | 55 | 0,00403 | 21,5 |  |
| M100Lb | 2,2 | 3 | 1350 | 13 | 0,97 | 76 | 15,6 | 0 | 1,7 | 4 | 125-156 | 70 | 0,00506 | 24 |  |

| MMDSG Single-phase cap start/cap run motors 1000 min-1 6 poles 50 Hz V=230V |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Size | Pn | Pn | n | Inom | cosφ | η | Mn | Mm/Mn | Ms/Mn | Is/In | Ca | Cm | J | Weight |  |
|  | kW | Hp | min-1 | A |  | % | Nm |  |  |  | μF | μ | Kgm2 | Kg |  |
| M80 | 0,37 | 0,5 | 920 | 3,3 | 0,94 | 51 | 3,84 | 0 | 1,5 | 2,1 | 63-80 | 16 | 0,00186 | 10,3 |  |
| M90S | 0,55 | 0,75 | 910 | 4,3 | 0,86 | 65 | 5,8 | 0 | 1,3 | 2,3 | 63-80 | 16 | 0,00228 | 12,6 |  |
| M90La | 0,75 | 1 | 945 | 5,8 | 0,88 | 64 | 7,6 | 0 | 1,3 | 2,7 | 80-100 | 25 | 0,00296 | 14,9 |  |
| M90Lb | 1,1 | 1,5 | 930 | 8,3 | 0,96 | 60 | 11,3 | 0 | 1,4 | 2,5 | 80-100 | 35 | 0,00267 | 17,4 |  |
| M100La | 1,1 | 1,5 | 935 | 8,1 | 0,91 | 65 | 11,25 | 0 | 1,5 | 2,3 | 100-125 | 35 | 0,00343 | 20,5 |  |
| M100Lb | 1,5 | 2 | 930 | 10,6 | 0,96 | 64 | 15,4 | 0 | 1,4 | 2,9 | 100-125 | 50 | 0,00403 | 21,8 |  |

Dimension Diagrams

Dimensions for CAP START / CAP RUN single phase motors B3 (B6-B7-B8-V5-V6)

| Size | A | AB | B | BB | C | D | E | H | HD | L | K | Y | AC | AD | AF | AP | AL | F | FB | GA | GD | DB |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| M63 | 100 | 120 | 80 | 106 | 40 | 11 | 23 | 63 | 187 | 240 | 7 | 12 | 123 | 124 | 92 | 121 | 19 | 4 | 15 | 12,5 | 4 | M4 |
| M71 | 112 | 136 | 90 | 108 | 45 | 14 | 30 | 71 | 203 | 280 | 8 | 12 | 147 | 132 | 92 | 121 | 24 | 5 | 20 | 16 | 5 | M5 |
| M80 | 125 | 160 | 100 | 125 | 50 | 19 | 40 | 80 | 235 | 290 | 9,5 | 17 | 165 | 155 | 119 | 146 | 17 | 6 | 30 | 21,5 | 6 | M6 |
| M90S | 140 | 175 | 100 | 131 | 56 | 24 | 50 | 90 | 249 | 322 | 9,5 | 17 | 181 | 159 | 119 | 146 | 22 | 8 | 40 | 27 | 7 | M8 |
| M90L | 140 | 175 | 125 | 156 | 56 | 24 | 50 | 90 | 249 | 347 | 9,5 | 17 | 181 | 159 | 119 | 146 | 22 | 8 | 40 | 27 | 7 | M8 |
| M100L | 160 | 200 | 140 | 170 | 63 | 28 | 60 | 100 | 269 | 393 | 11 | 21 | 198 | 169 | 119 | 146 | 36 | 8 | 50 | 31 | 7 | M10 |

Dimensions for CAP START / CAP RUN single phase motors B5 (V1-V3)

| Size | P | N | M | T | S | D | E | F | FB | GA | GD | L | LB | LA | AD | AF | AP | AL | AC | DB |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| M63 | 140 | 95 | 115 | 2,5 | 9,5 | 11 | 23 | 4 | 15 | 12,5 | 4 | 240 | 217 | 10 | 124 | 92 | 121 | 19 | 123 | M4 |
| M71 | 160 | 110 | 130 | 3 | 9,5 | 14 | 30 | 5 | 20 | 16 | 5 | 280 | 256 | 9 | 132 | 92 | 121 | 24 | 147 | M5 |
| M80 | 200 | 130 | 165 | 3 | 11,5 | 19 | 40 | 6 | 30 | 21,5 | 6 | 290 | 250 | 10,5 | 155 | 119 | 146 | 17 | 165 | M6 |
| M90S | 200 | 130 | 165 | 3 | 11,5 | 24 | 50 | 8 | 40 | 27 | 7 | 322 | 272 | 10,5 | 159 | 119 | 146 | 22 | 181 | M8 |
| M90L | 200 | 130 | 165 | 3 | 11,5 | 24 | 50 | 8 | 40 | 27 | 7 | 347 | 297 | 10,5 | 159 | 119 | 146 | 22 | 181 | M8 |
| M100L | 250 | 180 | 215 | 3,5 | 14 | 28 | 60 | 8 | 50 | 31 | 7 | 383 | 393 | 14 | 169 | 119 | 146 | 36 | 198 | M10 |

Dimensions for CAP START / CAP RUN single phase motors B14 (V18-V19)

| Size | P | N | M | T | S | D | E | F | FB | GA | GD | L | LB | AD | AF | AP | AL | AC | DB |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| M63 | 90 | 60 | 75 | 2,5 | M5 | 11 | 23 | 4 | 15 | 12,5 | 4 | 240 | 217 | 124 | 92 | 121 | 19 | 123 | M4 |
| M71 | 105 | 70 | 85 | 3 | M6 | 14 | 30 | 5 | 20 | 16 | 5 | 286 | 256 | 132 | 92 | 121 | 24 | 147 | M5 |
| M80 | 120 | 80 | 100 | 3 | M6 | 19 | 40 | 6 | 30 | 21,5 | 6 | 290 | 250 | 155 | 119 | 146 | 17 | 165 | M6 |
| M90S | 140 | 95 | 115 | 3 | M8 | 24 | 50 | 8 | 40 | 27 | 7 | 322 | 272 | 159 | 119 | 146 | 22 | 181 | M8 |
| M90LB | 140 | 95 | 115 | 3 | M8 | 24 | 50 | 8 | 40 | 27 | 7 | 347 | 297 | 159 | 119 | 146 | 22 | 181 | M8 |
| M100L | 160 | 110 | 130 | 3,5 | M8 | 28 | 60 | 8 | 50 | 31 | 7 | 393 | 333 | 169 | 119 | 146 | 36 | 198 | M10 |